Introduce

Kit Model: CTR-30 (three-color light) / CTR-30C (Colorful light)

Operating voltage: DC4.5-5V

Power supply: 3 AA batteries or USB charger

Dimensions: 60mm * 136mm * 60mm (L*W*H)

CTR-30 flash Christmas tree kit consists of three circuit board, allows 36 LED flash alternately, showing a Christmas tree in the space of three-dimensional profile (the night environment has better viewing).

Production steps:

Step 1:  Welding CTR-30A circuit board (referred to as A board)

- Soldering four resistors (R1 R3 R5 R7)

- Welding three resistance (R2 R4 R6)

- Welding three transistors, with horizontal installation (with directions)

- Welding three electrolytic capacitors, use horizontal mounting (with direction)

- Welding red LED (with direction)), the red LED numbered D1 ~ D6.

- After folded pin, inserted into the corresponding holes (note the mounting direction)

- The red LED welding finished (No. D1 ~ D6)

- Welding yellow LED (with direction), No. D7 ~ D12

- Welding green LED (with direction), number D13 ~ D18

- Now,  A board welding finish.

- Next using the same method welding CTR-30B circuit board , referred to as B board.

- A board and B board has different mounting positions of the two elements:

- 1. R2 R4 R6 resistance has different mounting position

- 2. The red, green and yellow LED has different mounting position

- After A board and B board both are welding finish, you can power test whether two plates normal light flashes. A, B board need to continue normal flashing and continue the following step.

- 11. Welding CTR-30C circuit board, called C board

- Welding a DC socket (with direction)

- Welding a key switch (with direction)

- A board and B board combination (with direction, the two plates are positioned like the arrow below), after two positioning arrows are aligned, use a soldering iron to put the two board together.

- Assembly the battery box and C board

- A. welding the wire battery case

- B. fixed the battery case with screws and nuts

- Combination of A, B, C board (with direction)

- Welding the top LED (with direction)

- Congratulation!! You finish the kit!!

- CTR-30 (three-color light)

CTR-30C (Colorful light)